E S T I M A T E

Abstract estimated cost for complete the following works of  VIVEKANANDA  EDUCATIONAL

SOCIETY  FOR  CHILDREN at Jagaddal, P. S.:-Sonarpur, Dist.:-South 24 Parganas.
=================================================================================

Rates are based on “P. W. D. (W. B.) Schedule of rates -1st August, 2002.”

=================================================================================

General building works of following items.

	Sl. No.
	Particulars of item of work
	Qnty & Unit
	Rate & Unit
	Amount

Rs.  P.

	
	
	
	
	

	1.
	Collapsible gate with 40 mm. X 40 mm. X 6 mm. tee as top and bottom guide rail 20 mm. X 10 mm. X 2 mm. vertical channels 100 mm. apart in fully stretched position 20 mm. X 5 mm. M.S. flats as collapsible bracings properly riveted and washered including 38 mm. steel rollers including locking arrangements, fitted and fixed in position with lugs set in cement concrete and including cutting necessary holes, chasing etc. in wall, floors etc. and making good damages complete.


	7.2 m²
	1500/=/ m²
	10800.00

	2.
	Simply iron fencing with locking arrangement after cutting out from pitters and refixing the same including mending all damages.


	14.40 m²
	110/=/ m²
	1584.00

	3.
	Wood work in door window frame, frame for stair case roofing.


	0.680 m³
	32458/=/m³
	22071.40

	4.
	Asbestos shed on stair case with wooden structure.


	
	L. S.
	6000.00

	5.
	Plaster with sand cement mortar (1 : 6).


	163.825 m²
	59/=/ m²
	9665.70

	6.
	Colour washing with pigments of any shade with a coat of white wash priming including cleaning and smoothing surface thoroughly of two coats of colour wash.


	368.020 m²
	926/=/% m²
	3407.90

	7.
	Art work with rule pointing after plaster of 25 mm. th. with sand and  cement mortar including admixture of pigment to match with colour of brick including raking out joints.


	8.620 m²
	113/=/ m²
	974.10

	8.
	Water supply and plumbing for drinking water.



	
	a) Tube well boring of depth 300m.


	
	L. S.
	21000.00

	
	b) 50 mm. dia. of G. I. pipe for piping with three numbers filter.


	
	L. S.
	30000.00

	
	c) 2 H. P. pump.


	
	L. S.
	9000.00

	
	d) 25mm. 15mm. 10mm. dia. of G. I. pipe.


	
	L. S.
	5500.00

	
	e) Over head tank of 750 liter.


	
	L. S.
	5000.00

	
	f) Basin shink rein water down pipe etc.


	
	L. S.
	8000.00

	
	g) Iron fitting.


	
	L. S.
	4000.00

	
	h) Labour charges.


	
	L. S.
	6000.00

	9.
	Painting with best quality synthetic enamel paint approved make and brand at wood and grill.


	160.590 m²
	41/=/m²
	6584.20

	10.
	Cost of drain work.



	
	a) Earth work in excavation in foundation in any kind of soil depth nit exceeding 1.5m.


	13.620 m³
	3418/=/%m³
	465.50

	
	b) Brick flat soling of single layer with picked jhama bricks at foundation and floor.


	15.525 m²
	105/=/ m²
	1630.10

	
	c) 100 mm. th. plain cement concrete ( 1:3:6) with 30 mm. to 40mm. size jhama khoa in column base.


	1.553 m³
	1818/=/m³
	2823.40

	
	d) First class brick work for foundation upto plinth of structure with cement sand mortar (1:6).


	8.438 m³
	1794/=/m³
	15137.80

	
	e) Plaster with net cement finishing.


	48.75 m²
	68/=/ m²
	3315.00

	11.
	Cost of boundary wall.



	
	a) Earth work in excavation in foundation in any kind of soil depth nit exceeding 1.5m.


	24.908 m³
	3418/=/%m³
	851.40

	
	b) Brick flat soling of single layer with picked jhama bricks at foundation and floor.


	28.500 m²
	105/=/ m²
	2992.50

	
	c) 100 mm. th. plain cement concrete ( 1:3:6) with 30 mm. to 40mm. size jhama khoa in column base.


	3.820 m³
	1818/=/m³
	6944.80

	
	d) First class brick work for foundation upto plinth of structure with cement sand mortar (1:6).


	16.760 m³
	1794/=/m³
	30067.40

	
	e) Plaster with net cement finishing.


	156.380 m²
	59/=/ m²
	9226.40

	=======================

Total            Rs.  2,23,041.60

=======================

Say            Rs.  2,23,000.00

=======================

	Estimated cost rupees two lacks twenty three thousand only.


Signature of authority.


       




Signature of Engineer.

E S T I M A T E

Abstract estimated cost for complete the following works of  VIVEKANANDA  EDUCATIONAL  

SOCIETY  FOR  CHILDREN at Sonajheel, Sonarpur, South 24 Parganas.
=================================================================================

Rates are based on “P. W. D. (W. B.) Schedule of rates -1st August, 2002.”

=================================================================================

General building works of following items.

	Sl. No.
	Particulars of item of work
	Qnty & Unit
	Rate & Unit
	Amount

Rs.  P.

	
	
	
	
	

	1.
	Shuttering and centering works with 25 mm. th. hard wood planks in chajja.


	4.050 m²
	123/=/ m²
	498.20

	2.
	Structural cement concrete M20(1:1.5:3) grade concrete in chajja with 20 mm down stone chips.


	0.405 m³
	3013/=/ m³
	1220.30

	3.
	M. S. reinforcement work of good quality including supplying, cutting, bending, binding, etc.


	0.80 Qtl
	2291.40/=/ Qtl
	1833.10

	4.
	Asbestos shed on stair case with wooden structure.


	
	L. S.
	12000.00

	5.
	Colour washing with pigments of any shade with a coat of white wash priming including cleaning and smoothing surface thoroughly of two coats of colour wash.


	459.56 m²
	926/=/% m²
	4255.50

	6.
	Art work with rule pointing after plaster of 25 mm. th. with sand and  cement mortar including admixture of pigment to match with colour of brick including raking out joints.


	5.40 m²
	113/=/ m²
	610.20

	7.
	Water supply and plumbing for drinking water.



	
	a) Tube well boring of depth 300m.


	
	L. S.
	21000.00

	
	b) 50 mm. dia. of G. I. pipe for piping with three numbers filter.


	
	L. S.
	30000.00

	
	c) 2 H. P. pump.


	
	L. S.
	9000.00

	
	d) 25mm. 15mm. 10mm. dia. of G. I. pipe.


	
	L. S.
	5500.00

	
	e) Over head tank of 750 liter.


	
	L. S.
	5000.00

	
	f) Basin shink rein water down pipe etc.


	
	L. S.
	8000.00

	
	g) Iron fitting.


	
	L. S.
	4000.00

	
	h) Labour charges.


	
	L. S.
	6000.00

	8.
	Cost of septic tank and soak pit.



	
	a) Earth work in excavation in foundation in any kind of soil depth nit exceeding 1.5m.


	10.123 m³
	3418/=/%m³
	346.00

	
	b) Brick flat soling of single layer with picked jhama bricks at foundation and floor.


	6.038 m²
	105/=/ m²
	634.00

	
	c) 100 mm. th. plain cement concrete ( 1:3:6) with 30 mm. to 40mm. size jhama khoa in column base.


	0.604 m³
	1818/=/m³
	1098.10

	
	d) First class brick work for 250 th. wall with cement sand mortar (1:6).


	4.081 m³
	1848/=/m³
	7541.70

	
	e) 125mm. th. cement brick work for partition walls in cement sand morter(1:4)


	2.700 m²
	258/=/ m²
	696.60

	
	f) Shuttering and centering works with 25 mm. th. hard wood planks roof slabs.


	3.319 m²
	123/=/ m²
	408.20

	
	g) Cement concrete M15 (1:2:4) grade concrete with 20mm. down stone chips at roof slab.


	0.604 m³
	3013/=/m³
	1819.90

	
	h) M. S. reinforcement work of good quality including supplying, cutting, bending, binding, etc.


	1 Qtl.
	2291.40/=/ Qtl.
	2291.40

	
	i) Plaster with net cement finishing.


	20.250 m²
	68/=/ m²
	1377.00

	
	j) Cost of soak pit with brickwork, R. C. C. cover slab, plaster and net cement finishing.


	
	L. S.
	8500.00

	9.
	Cost of retaining wall.



	
	a) Earth work in excavation in foundation in any kind of soil depth nit exceeding 1.5m.


	9 m³
	3418/=/%m³
	307.60

	
	b) Silver sand filling and spreading with local fine sand in between foundation and retaining wall.

 
	18 m³
	24640/=/%m³
	4435.20

	
	c) Brick flat soling of single layer with picked jhama bricks at foundation and floor.


	9 m²
	105/=/ m²
	945.00

	
	d) 100 mm. th. plain cement concrete ( 1:3:6) with 30 mm. to 40mm. size jhama khoa in column base.


	0.9 m³
	1818/=/m³
	1636.20

	
	e) First class brick work for foundation upto plinth of structure with cement sand mortar (1:6).


	9 m³
	1794/=/m³
	16146.00

	
	f) Plaster with net cement finishing.


	32.625 m²
	68/=/ m²
	2218.50

	10.
	Cost of drain work.



	
	a) Earth work in excavation in foundation in any kind of soil depth nit exceeding 1.5m.


	12.15 m³
	3418/=/%m³
	415.30

	
	b) Brick flat soling of single layer with picked jhama bricks at foundation and floor.


	13.5 m²
	105/=/ m²
	1417.50

	
	c) 100 mm. th. plain cement concrete ( 1:3:6) with 30 mm. to 40mm. size jhama khoa in column base.


	1.35 m³
	1818/=/m³
	2454.30

	
	d) First class brick work for foundation upto plinth of structure with cement sand mortar (1:6).


	6.75 m³
	1794/=/m³
	12109.50

	
	e) Plaster with net cement finishing.


	39 m²
	68/=/ m²
	2652.00

	11.
	Cost of boundary wall.



	
	a) Earth work in excavation in foundation in any kind of soil depth nit exceeding 1.5m.


	36.450 m³
	3418/=/%m³
	1245.90

	
	b) Brick flat soling of single layer with picked jhama bricks at foundation and floor.


	39.150 m²
	105/=/ m²
	4110.80

	
	c) 100 mm. th. plain cement concrete ( 1:3:6) with 30 mm. to 40mm. size jhama khoa in column base.


	4.212 m³
	1818/=/m³
	7657.40

	
	d) First class brick work for foundation upto plinth of structure with cement sand mortar (1:6).


	20.25 m³
	1794/=/m³
	36328.50

	
	e) Plaster with net cement finishing.


	175.50 m²
	59/=/ m²
	10354.50

	=======================

Total            Rs.  2,38,064.40

=======================

Say            Rs.  2,38,100.00

=======================

	Estimated cost rupees two lacks thirty eight thousand one hundred only.


Signature of authority.


       




Signature of Engineer.
