ESTIMATE AS PER PLAN FOR CONSTRUCTION OF V.E.S.C. AT JAGADDAL

SL. NO.
DESCRIPTION OF ITEMS
QTY.
UNIT
RATE
AMOUNT

1.
Earth work in excavation of foundation trenches in all sort of soil including attacking the spoil within a lead of 75 m. 
107.72
%m3
2973/-
3202.51

2.
Silver sand in filling in foundation trenches or plinth with silver sand in laver not exceeding 150 mm.
74.50
%m3
13242/-
9865.29

3.
Single brick flat soiling of picked jhama bricks including removing and dressing to proper leveling.
200.22
%m2
95
19020.90

4.
Cement concrete with hnama khow (25 mm to 30 mm, six) (10:36:18) excluding shuttering.
30.03
m3
1623
48738.69

5.
Bricks work in foundation upto plinth in cement mortar (1:6).
13.77
m3
1602
22059.54

6. 
D.F.C. with stone chips (6:2:1) double chequared complete 25 mm, th. 12.5 mm. th. first class brick work cement.
22.95
m2
90
2065.50

7.
Brick work in super structure upto roof in cement mortar (1:6)
58.40
m3
1650
96360.00

8.
Form work to slab, bin, column, lintel with 25 mm. th. wooden plank with necessary staging and striking out after completion of work complete.
750.00
m2
105
78750.00

9. 
R.C.C. ( 6:2:1) with 20 mm. graded stone chips as in column, lintel, slab etc.
101.57
m3
2373
241025.61

10.
M.S. reinforcement for all sort of R.C. work complete (a) mils steel
99.66
Qtl.
2083
207591.78

11
Artificial stone in floor dado staircase etc. with C.M. (4:2:1) with stone chips, 3 m. Th. sopping with 25 mm. Th.
357.24
m2
73
7044.50

12.
12 mm. th. cement planter (1:4) in internal and outside of wall.
1765
m2
48
84720.00

13.
Wood work in door and window frame fittings &  fixed complete including a protective sot of painting at the contract surface of the frame for sal.
.30
m2
28980
8694.00

ESTIMATE AS PER PLAN FOR CONSTRUCTION OF V.E.S.C. AT JAGADDAL

14.
25 mm. th. wodden shutter for door & window of shishu.
14.70
m2
1285
18889.50

15
125 mm. thick brick work with first class bricks in cement mortar (4:1)
93.31
m2
230
21461.30

16.
Grill for windows verandah
36.765
m2
360
13235.40

17.
Without integrated grills openable steel windows
36.765
m2
708
26029.62

18.
Supply best Indian sheat glass panels , 4 mm. th.
36.765
m2
376
13823.64

19.
(a) Lime tracing on roof with lime concrete (7:2:2) laid to proper slope thoroughly beaten & cured including top finishing ghoondies at junction with parapet etc. Shoping and finishing at mouths of rain-water pipes complete as per direction.         (b) 100 mm. th. (average) 
146.62
m2
201.00
29470.62

20.
Electrification & sanitary fittings complete.

L.S.

117701.47

21.
Plumbing including all fittings.

L.S.

47080.59

22.
Estimation is done on the basis of 1999-2000 P.W.D. schedule in order to take care normal price rice in building materials and laboure and other related, for the year 2001-2003 15%  is considered



141241.77






1247635.63

SIGNATURE OF ENGINEERS

